Protocol for sending methanolic suspensions for sugar and LLO analysis.

For analysis from MEFs, approximately 90% confluent, material from a single 15 cm dish or two 10 cm dishes (combined) is usually adequate (about 2 x 107 cells).

(1) Remove medium and wash each dish twice with ice-cold PBS, keeping dishes on ice.

(2) With dishes still on ice, add room-temperature methanol to each dish (5 ml per 10 cm dish, or 12 ml per 15 cm. dish).

(3)  Move dishes from ice to lab bench, and thoroughly suspend the cellular material with a scraper.

(4)  Using a pipet, transfer all of the suspension to a screw-cap plastic tube.

(5)  Keep at room temperature until ready for express shipping to Dallas.  Seal the tubes carefully, and ship at room temperature.